***************************************************************************

EMCP.DOC

***************************************************************************

This file contains documentation for EMCP, a version of Gabriele Gratton's

eye movement correction developed at the University of Illinois Cognitive

Psychophysiology Laboratory (CPL).

This version is based on EMCPX, which was written by James Turner, in 1993

for AIX and the IBM RS-6000.

Subsequent modifications were made by Brian Foote and Kevin Spencer.

Bill Gehring ported EMCPX to DOS using DJGPP, and added Neuroscan

data file support.  Brian Foote reincorporated this code into

the CPL's EMCP, and did additional ports to Borland C++ and Microsoft C++.

EMCP uses Neuroscan, Inc. EEG files (single-trial epoched

data produced by Neuroscan's Acquire and Edit programs).  It also requires

some other input files, one of which is optional. It corrects

the EEG data for vertical and horizontal ocular movement and blink

artifacts and produces an EEG file compatible with NeuroScan's analysis

software.   It is compatible with other data formats used

with previous versions of this program.

William J. Gehring

Human Brain Electrophysiology Laboratory

Department of Psychology

University of Michigan

Ann Arbor, Michigan 48109-1109

wgehring@umich.edu

April 16, 1996

Brian Foote

Cognitive Psychophysiology Laboratory

Department of Psychology

University of Illinois at Urbana-Champaign

603 E. Daniel

Champaign, IL  61820

(217) 333-3411

foote@p300.cpl.uiuc.edu

22 May 1996

***************************************************************************

The procedure is reported in:

Gratton, G., Coles, M. G. H., & Donchin, E. (1983).  A new

method for off-line removal of ocular artifact.  Electroencephalography

and Clinical Neurophysiology, 55, 468-484.

A distribution of this program for the RT11 operating system

FORTRAN IV is reported in:

Miller, G. A., Gratton, G., & Yee, C. M. (1988).  Generalized

implementation of an eye movement correction procedure.  Psychophysiology,

25, 241-243.

Documentation available with the Miller distribution goes into much more detail

than is possible here.  Part of that distribution is included here

(EMF11P.TXT and EMF11P.GID) and it includes some revision notes that

are informative.  Potential users should note that a number of conventions

have been adopted over the years (such as the blink detection criterion

EOGSEN, discussed in the Miller distribution).  Because the program has

evolved over many versions and operating systems, no one person remembers or

understands all of the conventions.  (Thanks to Greg Miller for permission to

include EMF11P.TXT and EMF11P.GID).

***************************************************************************

COMMAND LINE SYNTAX:

DJEMCP requires 2 input files: a cardfile that contains parameters

specifying input and output format, and a data file containing single

trials from an ERP experiment.  For NeuroScan users, this will be a .EEG

file.  It also allows you to specify whether the screen output will be

be verbose and whether to use a "Beckman calibration" (something that was

apparently necessary at the CPL but will be of no use to most users).

A final optional argument is the calibration file, which is documented below.

The version as compiled by DJGPP requires a DPMI host.  CWSDPMI is free

and included with djgpp (see http://www.delorie.com).  The easiest way

to get DPMI services without CWSDPMI is to run the program from a DOS window

in Windows 3.1.

the syntax is:

djemcp <+/->v<b> <parameter file> <data file> <calibration file>

(+v = verbose on, -v = verbose off, /b = Beckman calibration)

example as used in my laboratory:

cwsdpmi

djemcp +v s201.crd test.eeg

Output files:

The current version produces fewer output files than some versions

of EMCP.  In particular, corrected and uncorrected averages are not produced.

Appropriate lines of code in nsfiles.c can be un-commented and these will

be produced, but they will not be compatible with NeuroScan programs.

In the current version, if the input file is test.eeg, the

output files produced would be:

testc.eeg	corrected single trials

test.log	log file

***************************************************************************

TEST BATCH FILES

To test the programs on your system, you can use the batch files "dj.bat" and "wat.bat"

in the testdata directory.  The single trial output files will be comparable to c1s12l01.sng

(see "test" documentation below).

***************************************************************************

NOTES ABOUT THE CURRENT VERSION

1)  The current version was written to retain compatibility with previous

data formats.  For the analysis of Neuroscan files, however, much of the

input cardfile is not necessary.

2)  I have included some files that you may use to test modifications

of this code, to ensure that the output is the same as

produced by previous versions.  The tests that were performed are documented

below.

3)  This version was compiled and tested using djgpp version 2, an MS-DOS

port of the gnu gcc C compiler that requires DPMI services. It is

on the world wide web at http://www.delorie.com. Use of this

required adding "__attribute__ ((packed))" in a number of places in the

header file sethead.h (available from NeuroScan).  Note that djgpp version 1

is not recommended because it chokes on the command line arguments used with

emcp.

DPMI services can be provided either by running the program in a

DOS window of Windows 3.1 or by running the program CWSDPMI before

running DJEMCP.  CWSDPMI can be obtained at the website above.

4) NeuroScan EEG data files are in short format, and integer overflows can

happen after correction when the data (stored as floats) are

converted back to short integers, e.g., if the digitizer was close to

saturated before correction and then correction pushed the

data over the top.  Output data are scanned for overflow in nsfiles.c

If overflow is detected, program tries to subtract a baseline consisting of

the average activity during the entire epoch.  If there is still an

overflow, the trial is rejected (rejection flag set to 6 and set all the data

set to 0).  These trials will show up as rejected in NeuroScan's EDIT

module.

5) For all input formats except NeuroScan, trials that are rejected because

of flat or out-of-scale EEG or EOG are not written to the output file.  For

the NeuroScan input format, however, >all< trials are written to the output

file.  Trials with flat or out-of-scale data are marked as "rejected" and

will show up that way in the EDIT program.  Keeping the bad trials in the

output is done for a couple of reasons:  sequential analyses where one trial

is classified on the basis of the behavior on previous trials may depend on

non-EEG information (e.g. squeeze force) on the previous trial.  Also,

output files produced this way are exactly the same size as EEG files,

which provides a check on the success of the EMCP run.

6)  If the number of sweeps is too large for current memory limits, the

program will crash.

7)  Other causes of crashing may include incorrect input parameters.

8)  Three compiled versions are included:

Gehring's DJEMCP version was compiled with djgpp (a port of the gnu

gcc comipiler to DOS) and tested on a Dell Optiplex 90 MHz Pentium

with 16Mb RAM.

EMCP.EXE was compiled with Watcom version 11.

Both versions have been tested under Windows NT 4.0

Note:  10/11/99:  Borland and Visual C/C++ compiled versions are no longer included.

***************************************************************************

Assumptions of the current version:

1)  The EEG file input filename should have no more than 7 letters in the

prefix.  Output files will append that prefix with a "c" to denote

that the data have been corrected.  Thus, DATA001.EEG will yield

the output file DATA001c.EEG.

2)  For EEG files, the current version assumes that one channel

in the input is called "VEOG" and another is called "HEOG" and a third

is "FZ." For other formats, this information is defined in the header

file emcp.h.  If your EEG file does not have the proper names, the

defaults in emcp.h will be used.  (You can also change electrode names

using the NeuroScan's EDIT program.)

***************************************************************************

FILES

The files contained with this distribution are listed below.

djemcp.exe      - version compiled with djgpp (see http://www.delorie.com)

runs under DOS with CWSDPMI or under Windows 95/98/NT

emcp.exe        - version compiled with Watcom version 11:  runs under Windows 95/98/NT

allocate.c	- used for allocating array space in the current version

allocate.h

allocate.o

eegtoasc	- separate parogram provided by NeuroScan for converting

eegtoasc.c		.EEG files to ASCII format

eegtoasc.exe

feegtoas	- used for testing.  Takes an EEG file in float format

feegtoas.c		and converts to an ascii file.

feegtoas.exe

makefile	- 	- makefile for DJGPP version

compute.c	- computational functions

emcp.c	- 	- emcp main program

emcp.doc	- this documentation file

files.c		- I/O code

The following two files were adapted directly from Gabriele Gratton's fortran

selection card routines on AIX:

scard.c		- used for reading selection cards in parameter file

- Contains documentation of selection card format.

scard.o

selectc.c		- used for reading selection cards in parameter file.

selectc.h

This file is available from NeuroScan, Inc.

sethead.h	- modification of the sethead.h header file available

from NeuroScan, Inc.  Describes the headers

in the .EEG files.

sethead.old	- NeuroScan's original sethead.h file

* build files

*

********************************************************************

PARAMETER FILE:

For non-SYNAMPS formats, the input parameter file (or "card file")

contains important information about the input and output data formats.

The logfile will contain much of this information as read by the

program -- useful for debugging purposes.

The parameters marked * can be set to 0 for .EEG files derived from

a NeuroScan system.  Parameters marked # are additional parameters

that can be set to 0 for SYNAMPS systems.

Here is a description of the information in c1s12ns.crd.  (see TEST 2

above).  Note that the actual file should be one value per line.

Line	Cardfile	Variable		description

no.	Value		in Program

--------        ----------              ------------------------

1	440*	parameters.trialLength		points per waveform

2	3* 	parameters.numIDs		number of ids w/ each wave

3	12* 	parameters.totalChannels	total number of channels

4	10*	parameters.numEEGChannels       number of eeg

channels (after 2 EOG)

5	12 	parameters.mastoidOpt           channel number of

mastoid channel to use for

average reference (0 = don't

subtract mastoid).  Numbering

begins with 1.  1/2 of this

value at each data point is

subtracted from each EEG

channel,  thus assuming

that the data were collected

with the other mastoid

as reference).

6	5*	parameters.digitizingRate	digitizing rate (ms per point)

7	2.70*	parameters.EOGSensitivity	EOG channel sensitivity

(A/D per uVolt) (0 means

get from NeuroScan file)

8	2 	parameters.singleTrialOpt

0=correct & output only trials that fit

a selection card

1=correct and average only/no singletrials

2=correct & output all single trials

9	0 	parameters.vectorsOpt		0=full id vectors in output

10	12# 	parameters.ADBits		A to D Resolution in Bits

(probably 12 or 16)

11	0 	parameters.inputFormat		0=Neuroscan, 1=CPL, 2=PC

12	1 	parameters.inputType		1=binary or NeuroScan  2=ascii

13	0 	parameters.outputFormat		0=NeuroScan, 1=CPL,  2=PC,

3=binary

14	9.3f 	parameters.outputSpecification	For ASCII output e.g. 7.1f

15	15 	parameters.numTrials		Number of trials, or:

0=find number of trials by reading,

-1 = get from NeuroScan header

16	EMG	 				Channels to leave alone,

ONLY VALID FOR NEUROSCAN

INPUT FORMATS

(include a dummy string here

otherwise).  Separate each by

a space (see example below).

17+	[i2=21]					Selection cards:  determines

[i2=22]	                                averaging bins and also

[i2=23]                                 determines which event-locked

average will be subtracted

from each single trial.

*****************************************************************************

CHANNEL SPECIFICATIONS

The traditional method for specifying channels was for EMCP to assume that the

first 2 channels were ocular movement channels, some number of other channels

were EEG channels to be corrected, and the remainder should be left alone

(useful for EMG, analog response devices, and stimulus markers).  The

parameters above are set up to be consistent with this early format:

Total Channels is the total number of Channels, and Number of EEG channels

indicates the number of channels to correct (following the first 2 EOG

channels).  Exactly which channels are the EOG channels are now specified in

the file emcp.h.  (Any of the first "Number of EEG Channels"+2 can be VEOG

or HEOG).

For NeuroScan input format, however, DJEMCP assumes that all channels will be

corrected except for those whose labels appear on the "Channels to be left

alone" line  (see SYNAMPS example below).  It assumes that vertical eye

movement has the VEOG label and horizontal has an HEOG label.  It also

requires a channel with the FZ label.  (That channel is used to recover some

out-of-scale EOG values and thus it's best if it really is Fz).

*****************************************************************************

EOG SENSITIVITY

The sensitivity of the EOG channel, in A/D per microvolt, is used to compute

a criterion that determines whether EOG activity is classified as a blink or

as a saccade.  Note that a slope is determined using this value multiplied

a scaling factor that is 14.0 in the present version. Greg Miller's release

of EMCP for RT11 contains extensive documentation on this and its history.

If this value is specified in the NeuroScan file, this parameter should be

set to 0 in the cardfile.

*****************************************************************************

ALTERNATIVE INPUT FORMATS.

Two different types of input formats besides NeuroScan .EEG can be used,

and each can be in binary or ascii format.  Note that these are normally only

useful for analyzing the data in some other system -- analysis with NeuroScan's

software only requires .EEG input and output formats.

1)  CPL format.  This format stores a string of identification values with

each channel.  The file is arranged as follows, for a file with 3 IDs and

440 points per channel, with two trials and 3 channels:

Trial 1 Chan 1: 	ID1 ID2 ID3 Point1-Point440

Trial 1 Chan 2:		ID1 ID2 ID3 Point1-Point440

Trial 1 Chan 3:		ID1 ID2 ID3 Point1-Point440

Trial 2 Chan 1: 	ID1 ID2 ID3 Point1-Point440

Trial 2 Chan 2:		ID1 ID2 ID3 Point1-Point440

Trial 2 Chan 3:		ID1 ID2 ID3 Point1-Point440

The program EEGTOASC creates a file in this format from a

NeuroScan EEG file.  The IDs created are:

ID1 = Trial

ID2 = Trial Type (can be used for averaging bin definitions)

ID3 = Channel Number

An example is the c1s12l01.asc file provided with the test program

(see TEST #1).

2)  PC format.  This format stores a string of identification values with

each channel.  The file is arranged as follows, for a file with 3 IDs and

440 points per channel, with two trials and N channels:

Point 1:	ID1 ID2 ID3 Channel1 Channel 2 Channel 3

Point 2:	ID1 ID2 ID3 Channel1 Channel 2 Channel 3

.

Point 440:	ID1 ID2 ID3 Channel1 Channel 2 Channel 3

3)  .EEG input but CPL or PC output.

If you use .EEG as input and produce one of the following output formats,

the IDs will correspond to the 3 IDs indicated in #1 above.

***************************************************************************

SELECTION CARDS.

Selection cards are used to describe the averaging bins. Traditional

use of EMCP created average ERPs according to these bins, but this

facility is commented out in the present version.  These averaging bins

are still important however, because they determine the average waveform that

is subtracted from each single trial before correction factors are computed.

(The idea is that the correction factors should not be influenced by

event-related ERP activity).  In older formats, defining the bins was handled

by "IDs" and thus the selection card format is based on a format like I1=1,

meaning that trials where the value of ID1 is 1 will be included in the

averaging bin.

The selection card format is documented in scard.c and selectc.c. The format

"i2=21" says that the trials where ID #2 is 21 (or in the case of neuroscan

files where the Trial type code is 21) will all be grouped into one average.

What I normally do is determine the most important experimental manipulations

and supply EMCP with selection cards that will describe those as separate

bins.  In the example above, I2=21 could be tones with probability .20,

I2=22 could be tones with probability .30 and I2=23 could be probability .50.

In djemcp, Neuroscan ID and event codes are converted internally to ID codes.

Selection cards can be specified by the trial number using the first ID

value (e.g., I1<10) or by the event code number (I2,as in the previous

paragraph).

****************************************************************************

A TYPICAL RUN:

Here is an example cardfile that we're using with our SYNAMPs system.  Note

that some of the values are replaced by information in the NeuroScan headers.

For a sample run, input and output files see "Test 4" below.

0

0

0

0

0

0

0

2

0

0

0

1

0

9.1f

-1

EMG-L EMG-R

[i2<10]

[i2=111|i2=122|i2=133|i2=144|i2=155]

[i2=115|i2=125|i2=135|i2=145]

[i2>100&i2<200]

Command line syntax:

djemcp +v s201.crd s211od3.eeg

***************************************************************

Calibration file:

The program CAN (optionally) use a calibration file, specified

on the command line, where each line in the file represents the

A/D points per microvolt for the corresponding channel.  Only EEG and

EOG channels (as specified by the numEEG.Channels value in the card file)

are calibrated in this manner.  Normally with SYAMPS you won't use

this because of the internal calibration, and NeuroScan provides

ways for you to take advantage of internal calibration with

non-SYNAMPS systems.  DO NOT USE THIS OPTION IF YOU INTEND TO

KEEP THE ANALYSIS WITHIN THE NEUROSCAN FORMAT.  THE PRESENT VERSION DOES

NOT PRODUCE EEG FILES WITH THE CORRECT CALIBRATION GIVEN AN EXTERNAL

CALIBRATION FILE. It should not be difficult to modify the program to do

this, but there has been no need for this yet. In other words,

only use this option if you intend to produce a CPL or PC version

output file.

********************************************************************

TESTS

The following files were provided by Leun Otten of the

Cognitive Psychophysiology laboratory for testing purposes.

The input files are ASCII and .EEG format data from their

neuroscan system.  The output files are those created with

the emcpx program (as of August 1995) as run on CPL's RS-6000

and as such should be considered the Gold Standard for verifying

the accuracy of any modifications to this version of emcp.

c1s12l01.eeg	input .EEG file

c1s12l01.txt	input single trial ASCII file produced by

converting c1s12l01.eeg to ascii using

eegtoasc.exe (program provided by neuroscan)

c1s12.cal	input calibration file, A/D points per microvolt for

each channel

c1s12.crd	input card file, describing input parameters

c1s12l01.cav	output corrected averages

c1s12l01.log	output log file

c1s12l01.out	screen output produced by emcpx

c1s12l01.rav	raw (uncorrected) averages

c1s12l01.sng	corrected single trials in ascii float format

c1s12l01.asc	input file in ascii format that is equivalent to

c1s12l01.txt

note:  PC version does not produce average output files (.cav or .rav)

*************************************************************************

Note that the tests below all made use of calibration information

provided by the calibration file.  This is not normally necessary

with NeuroScan's SYNAMPS system because calibration is performed by

ACQUIRE and the calibration factors as stored with the file are used by

emcp.

*************************************************************************

TEST 1:  ASCII to ASCII

To test the program, the first comparison showed the

ability of the program to take the ascii file c1s12l01.txt

and produce the same outputs as CPL's emcpx.

Because c1s12l01.txt has 8 letters in the prefix, I used

a copy of it named c1s12lt.txt for testing.  I also had

to modify the cardfile to be compatible with the

current input format.

This test was performed with the following commands:

cwsdpmi

copy c1s12l01.txt c1s12lt.txt

djemcp +v c1s12new.crd c1s12lt.txt c1s12.cal

Input:

c1s12new.crd	a modification of c1s12.crd for compatibility

with current parameter format

c1s12lt.txt	c1s12l01.txt renamed to 7 letter prefix.

c1s12.cal	same calibration file as used by CPL's emcpx

Output:

c1s12lt.txt	input file

c1s12lt.log	log file

c1s12lt.sng	single trials output

Note:  The batch files "dj.bat" and "wat.bat" in the testdata

directory perform this test, producing the files dj.sng and wat.sng.

*****************************************************************

TEST 2:  EEG to EEG

The next step was to test the capability of the program to take

Leun's EEG file and produce an EEG file equivalent to the

ASCII file produced by CPL's emcpx.  This checks the conversion

from EEG format to the format used by emcpx.  To do this, I had

to

1 - modify emcp to produce float (rather than short) binary

output.  This was done using the presently commented-out

code "special purpose for test" in files.c

2 - run that output through feegtoas.c to create an ascii version

3 - compare that to the single trial output c1s12l01.sng.

This test was performed with the following commands:

copy c1s12l01.eeg c1s1201.eeg

cwsdpmi

djemcp +v c1s12ns.crd c1s1201.eeg c1s12.cal

mv c1s1201c.eeg c1s1201f.eeg

feegtoas c1s1201f.eeg c1s1201a.eeg

Note that the output is scaled differently than the input, because

of the calibration (which does not end up being correct).

Input:

c1s12ns.crd	card file modified for this analysis

c1s1201.eeg	input EEG file - a copy of c1s12l01.eeg

renamed to a shorter prefix.

c1s12.cal	same cal file as before

Output:

c1s1201.log	log file

c1s1201f.eeg	eeg file output in float format (not compatible

with neuroscan).

Other:

c1s1201a.eeg	float format EEG after conversion to ascii

(for comparison to c1s12l01.sng)

c1s1201c.eeg    regular Neuroscan-compatible output file, from

a separate run using the usualy short integer output

(using the djemcp command line above).

*****************************************************************

TEST 3:  EEG to ASCII

This tests the capability of the program to take a .EEG input file

and produce an ASCII output equivalent to that produced by emcpx.

Commands:

cwsdpmi

djemcp +v c1s12n-c.crd c1s1201.eeg c1s12.cal

Input:

c1s1201.eeg	a copy of c1s12l01.eeg

c1s12n-c.crd	card file for this analysis

c1s12.cal	same calibrations as before

Output:

c1s1201c.asc	corrected single trials in ascii format (renamed

after the run from c1s1201.sng).

Note that (as compiled by djgpp) djemcp produces ascii output that

is slightly different from c1s12l01.sng (the output of emcpx as compiled

on the RS/6000 AIX system).  In particular, line 714 of the PC output

file "c1s1201c.asc" has 22.806 where the AIX output file "c1s12l01.sng"

has the value 22.805.  This difference is due to an apparent difference in

the rounding convention adopted by printf function in each of the two

compilers.  the actual number is 22.80550 in both cases.  djgpp uses IEEE

standard 754, which is described in this excerpt from the mail archives:

printf will "...round correctly after IEEE standard 754, which means that

numbers ending with 5 are rounded to the next even number.

So 1.15 will round to  1.2, 1.25 will round to 1.2 and 1.35 will round

to 1.4. (Of course 1.250000001 will still round to 1.3)."

Other rounding differences are apparent.  These are listed below:

Line 1405  DJEMCP:  0.000;  EMCPX:  -0.000;  Actual:  -0.00003

Line 3337  DJEMCP: -25.578; EMCPX: -25.579;  Actual: -25.57850

Line 4762  DJEMCP: -0.001;  EMCPX:  -0.000;  Actual:  -0.00089

*****************************************************************

TEST 4:  Normal run EEG to EEG.

This is not a comparison to Leun's data, but rather a demonstration

of a typical run of EMCP, without the calibration factors.  The input

file is the same 15 trial .EEG file, c1s1201.eeg, renamed as nocalib.eeg.

In this case the output is scaled the same as the input.  Note that with

these very few trials, djemcp does reasonable blink correction, but that

it does not deal very well with the one horizontal eye movement.

Commands:

cwsdpmi

djemcp +v nocalib.crd nocalib.eeg

Input:

nocalib.eeg	a copy of c1s12l01.eeg

nocalib.crd	card file for this analysis

Output

nocalibc.eeg	corrected single trials, compatible with SCAN.

nocalib.log	log file